| Document Number | UM-64NNP-V1_REWORK1 |
| --- | --- |
| Author | Alex Campean |
| Client | University of Michigan / Chestek Lab |
| Project | UM-64NNP-V1 |
| Statement of Work | - Remove U16 and replace with Texas Instruments SN74LVC1G32DRYR or equivalent OR gate. - If using hot air, a silicone cover is recommended to protect the flex circuit from damage. - Recommended re-work settings are 275⁰C hot air, 160⁰C pre-heat, A30 (3rd to last setting) air flow. - Cut the EMG_SCK trace going to U17, pin 6 (D Flip Flop CLRn input pin) as shown. - Wire an Onsemi NC7SV14P5X or equivalent inverter as shown. - Pin 2 = EMG_SCK - Pin 3 = GND - Pin 4 = U17 pin6 (CLRn) - Pin 5 = DVDD |

Description

Remove U16 and replace with SN74LVC1G32DRYR (replacement part not shown) Silicone cover to protect flex area.

Description

Cut EMG_SCK trace here From: EMG_SCK To: U17 pin6 (CLRn) NC7SV14P5X 5 = DVDD 4 = U17 pin6 (CLRn) 3 = GND 2 = EMG_SCK SOLDER SOLDER SOLDER SOLDER SOLDER SOLDER