| Document Number | UM-64NNP-V1_REWORK2 |
| --- | --- |
| Author | Alex Campean |
| Client | University of Michigan / Chestek Lab |
| Project | UM-64NNP-V1 |
| Statement of Work | - Remove FB1,FB2, FB3, FB4 and replace with 0402 zero-ohm resistors (Panasonic ERJ-2GE0R00X). - Remove R101, R201, C103, C203. - Add 1k, 0402 resistor (Panasonic ERJ-2RKF1001X) as R27. - Add 49.9k, 0402 resistor (Panasonic ERJ-2RKF4992X) as R28. - Add 2x 22uF, 0402 capacitors (Murata GRM158R61A226ME15J) to C103 and C203. Note that the capacitors must be stacked on top of each other. |

| Description |  |
| --- | --- |
| FB1,FB2 FB3,FB4 | Remove FB1, FB2, FB3, FB4 and replace with 0402 zero ohm resistor. |

| Description |  |
| --- | --- |
| R201 R101 C103 C203 | Remove R101, R201, C103, C203. |
| C103 C203 R27 R28 | Add 1k, 0402 resistor as R27. Add 49.9k, 0402 resistor as R28. Add 2 stacked 22uF, 0402 capacitors to C103, C203. |

| Description |  |
| --- | --- |
|  | Jumper a wire across U101 and U201 as shown. Make sure the wire does not touch any other board metal. Only U101 shown. |